| # Lab Operation Hazard Analysis |  |  |  |  |
| --- | --- | --- | --- | --- |
| # Task | Potential Hazard | ## Condition Triggering ## Hazard into Incident | Lab Procedural Requirements | Safety and PPE Requirements |
| 1) Inspect and set up drill press. | Defective chuck, switches, clamps, cord, etc. | Operating drill press with defective parts. | Do not operate drill press if defective. Attach “Do Not Operate” tag. |  |
| 2) Center punch holes at both ends of work-piece. | Work-piece not securely clamped while center-punching holes. | Work-piece slipping from underneath punch when punch is hit with hammer. | Work-piece is to be secured in vise before striking punch. Select No. 2 combination drill and countersink. |  |
| 3) Drill and countersink center holes on drill press. | Placing work-piece in position for drilling without clamping in vise. | Bringing drill in contact with work-piece. | Work-piece is to be secured in vise before drilling holes. |  |
|  | Leaving chuck wrench in chuck at time when machine is started. | Starting drill press. | Chuck wrench to be removed from prior to starting machine. |  |
|  | Stop/start buttons not located within easy reach of operators, causing student to reach behind or alongside moving part or not being able to shut off machine in an emergency. | Starting machine; having to shut machine off (normally), having to shut machine off in an emergency. | Stop/start button to be located within easy access of operator. | Start/stop buttons identified with bright colours. |
|  | Students with long hair, long sleeved shirt, baggy clothing, drawstrings on hooded sweater | Clothing, hair, etc. coming into contact with drill or chuck while machine is in operation. | Sleeves rolled up, hair in nets or tied back, baggy clothing and drawstring secured. |  |
|  | Rotating drill in chuck. | Students’ hand or arm coming in contact with spinning drill. | Student to keep full concentration on operations. |  |
|  | Drill breaking during drilling mode. | Excessive pressure placed on drill and/or not lubricating drill. | Drill should not be jammed down into stock, use slow and even pressure; lubricate. | Face shield and/or goggles. |
|  | Flying metal chips. | Working on drill press. | If excessive chips begin to fly from work-piece, shut off machine and ask for assistance. | Face shield and/or goggles. |
|  | Metal chips created during drilling procedure. | Student attempting to remove chips while machine is running. | Drill press is to be shut off and chips cleaned with brush. |  |
|  | Drill sticks in work. | Drilling stock. | Stop machine, free by hand. |  |
|  | Pinch points at belts. | Adjusting belts. | Always stop press before adjusting belts. |  |